����������������������������������������������������������������������������ͻ

�         HWiNFO � Copyright (c)1995-2003 Martin Mal�k - REALiX(tm)          �

� �������������������������������������������������������������������������� �

�                # Version 4.8.8; Last Update: Jan-21-2003 #                 �

����������������������������������������������������������������������������ͼ

������������������������������������������Ŀ

�  USE THIS PROGRAM ONLY AT YOUR OWN RISK  �

��������������������������������������������

CONTENTS

1.  Intro

2.  Updates and download

3.  Requirements

4.  Utilities distributed with HWiNFO

5.  Registration

6.  Creating a logfile

7.  Limitations

8.  Running HWiNFO under V86 Memory Managers and Windows

9.  Resetting the CPU

10.  HWiNFO Crash

11.  Secondary Video Info

12.  PCI chipsets comparison

13.  Hardware monitor chip support

14.  IDE Disk Transfer Modes

15.  Miscellaneous notes

16.  Some abbreviations

17.  Codenames

18.  Thanks

19.  Disclaimer and Terms of License

## 1. Intro ##################################################################

HWiNFO is a *POWERFUL* hardware detection program. It is simple, quick and

on most systems very efficient. It "tries" to give only important information

about all the hardware devices in your computer so that you need not look

inside the PC to see what's there.

## 2. Updates and download ###################################################

You can contact the author at: martin@hwinfo.com

or: malik@svf.stuba.sk

The newest version is always (immediately) available at:

HWiNFO homepage - http://www.hwinfo.com

SAC ftp       - ftp.elf.stuba.sk/pub/pc/utildiag

SAC WWW       - www.sac.sk

At other sitez/BBS later...

SimTel        - ftp.simtel.net/pub/simtelnet/msdos/sysinfo

SAC mirrors   - ftp.sac.sk/pub/sac/utildiag

ftp.cdrom.com/pub/sac/utildiag

ftp.uakom.sk/pub/mirrors/sac/utildiag

ftp.vse.cz/pub/msdos/SAC/pc/utildiag

ftp.bke.hu/pub/mirrors/sac/utildiag

esca.atomki.hu/paradise/sac/utildiag

ftp.externet.hu/pub/mirror/sac/utildiag

ftp.osb.hu/pub/mirrors/sac/utildiag

ftp.cs.tu-berlin.de/pub/msdos/mirrors/stuba/pc/utildiag

ftp.pwr.wroc.pl/pub/pc/sac/utildiag

cert.unisa.it/pub/PC/SAC/utildiag

ftp2.itb.it/pub/PC/SAC/utildiag

ftp.comp.hkbu.edu.hk/pub/sac/utildiag

ftp.nsysu.edu.tw/PC/SAC/utildiag

## 3. Requirements ###########################################################

- 80386 or better CPU

- cca. 500 KBytes of free System Memory

- cca. 1.5 MBytes of free XMS Memory (XMS manager recommended)

## 4. Utilities distributed with HWiNFO ######################################

There are four utilities distributed along with HWiNFO:

GETBIOS.COM     This utility retrieves all BIOS images and saves them to

files <BIOSseg>.ROM

This program doesn't run in V86 mode in order to avoid

BIOS relocation.

GETDDC.COM      This utility saves VESA Display Data Channel data to file

DDC.DAT if your video card and monitor support DDC

GETPCI.EXE      This utility stores all data from PCI configuration space

registers into files <VendorID><DeviceID>.PCI

GETPNPRS.EXE    This utility saves resource data for all Plug-and-Play

capable devices into files PNP<CSN>.RES

GETSMBUS.EXE    This utility reads data from all devices connected to SMBus

(e.g. HW monitors, P.I.ROM, Scratch EEPROM, SPD DIMM...)

and saves to files SMBUS<ADR>.DAT

These utilities are free and are designed to help me improve HWiNFO if

you send me the results.

## 5. Registration ###########################################################

HWiNFO is shareware, meaning that you can test the program for free whilst

checking if it is useful for you. The public (unregistered) version has all

the features as the registered version except the "Create report file"

feature. The other disadvantage is the message that HWiNFO isn't registered

and that you have to press a random key while the results are shown.

If you think you could use this program and don't want to see that stupid

message, or if you continue using it after a trial period of 14 days, you

should register. Otherwise after this period HWiNFO becomes an illegal copy

and you have to delete it (ho-ho-ho :-)).

If you decide to register, you should print out the file REGISTER.FRM,

fill it and send to the address in the file.

If you include your Internet e-mail address, you will receive the

key file via e-mail, otherwise snail mail.

Once you have obtained the key file needed to register, copy it to the

HWiNFO directory and the current version will become registered. Of course,

you mat not distribute the key file, it's only for your personal use.

Well, the Registration fee is:

�������������������������������������ͻ

�  Non-Commercial  �    Commercial    �

�������������������������������������͹

� DEM � US$ � Sk/KC� DEM � US$ � Sk/KC�

�����������������������������������������������������͹

� 1 User        �  35 �  20 �  500 �  70 �  40 � 1000 �

�����������������������������������������������������Ķ

� 2-10 Users    �* 30 �* 16 �* 450 �  60 �  32 �  900 � per copy

�����������������������������������������������������Ķ

� 11-20 Users   �* 25 �* 13 �* 400 �  50 �  26 �  800 � per copy

�����������������������������������������������������Ķ

� 21-50 Users   �* 20 �* 10 �* 300 �  40 �  20 �  600 � per copy

�����������������������������������������������������Ķ

� 51-100 Users  �* 15 �*  7 �* 200 �  30 �  14 �  400 � per copy

�����������������������������������������������������Ķ

� UNLIMITED     �Contact the author�Contact the author�

�����������������������������������������������������ͼ

* for schools, educations, etc.

When registering with a check or wire transfer please add 10% to

the total price to cover bank transfer and conversion costs.

For any special requirements contact the author.

A registration is valid for all future versions.

And there will be a lot of newer versions of course.

Please read Terms of License at the end of this file.

You should register NOW !

## 6. Creating a logfile #####################################################

There are three possible methods how to create a logfile:

1. Simple log

After pressing the <F2> key, HWiNFO adds the results from current screen

to a logfile.

2. Report file ("-r" switch) [REGISTERED VERSION ONLY]

HWiNFO creates AUTOMATICALLY a COMPLETE logfile at startup.

This logfile has the same format as the one created with <F2> key, but

there's no need to navigate HWiNFO through all the screens.

3. Short report file ("-s" switch) [REGISTERED VERSION ONLY]

HWiNFO creates AUTOMATICALLY a logfile, which contains only the

MOST IMPORTANT items. This format is suitable for further automatical

processing (or printing) of logfiles (e.g. if you would like to create

an inventory of more machines). Here's a sample of the format:

CPU=Intel Pentium III Xeon, 550.0 MHz

Bus=ISA + PCI + A.G.P.

Chipset=Intel 82440GX AGPset

Memory=128 MBytes

...

VideoChipset=nVIDIA GeForce 256 DDR

VideoMemory=32768 KBytes of SGRAM

3Daccel=3Dfx Voodoo 2

...

If you use a capital letter (either "-R" or "-S") the benchmark will not

be performed.

The "-v" switch enables Verbose Mode, where all screen outputs are supressed.

The default log filename is "HWiNFO.LOG". You can change its name if you

enter it as a paramater to HWiNFO (e.g. "HWiNFO.EXE -S -v HOME.LOG").

Samples of logfiles can be found at HWiNFO homepage.

## 7. Limitations ############################################################

The Mainboard Chipset identification works only on newer (� 1991 and above)

AMI and newer AWARD BIOSes. HWiNFO is also able to detect some chipsets, not

depending on the type of BIOS (see HWiNFO.LST).

Fixed Disk Buffered Data Transfer rate reads data from one physical sector

until 4 seconds. Some caches (like Hyperdisk; but NCACHE or SMARTDRV don't)

could cause invalid results. Also some mainboards, which have incompatible

timers could cause problems.

Fixed Disk Average Seek receives results while seeking permanently 100* from

cylinder 0 to a cylinder, which is 1/3 of the max. cylinder number. Thus some

programs could report another result if they seek to an another cylinder.

SCSI disks and some Cache programs (Hyperdisk) don't generate Seek requests,

thus the Seek and RPM result is abnormal and HWiNFO displays <N/A>.

PCMCIA detection requires Socket Services or Card Services to be installed.

If you want more info about sockets, install Card Services.

The VESA Local Bus can be detected only if a VLB card is present.

CD-ROM benchmark requires MSCDEX.EXE (Microsoft CD-ROM Extensions) to

be installed and an inserted data CD.

## 8. Running HWiNFO under V86 Memory Managers and Windows ###################

In order to achieve best results, HWiNFO accesses directly the hardware.

Some of the routines (privileged) require to be run in Real mode or at

privilege level 0 (PL0, most privileged), but in V86 mode are all

applications run at PL3. This means, if such a routine is run, the CPU

raises an exception, which is handled by the V86 manager. Note, that

V86 mode is the only way to run a DOS application (Real mode) under

MS Windows (Protected mode). EMM (Expanded Memory Manager) also use the

V86 mode to provide paging.

But HWiNFO v3.00+ is able to run the privileged routines in V86 mode under

some circumstances. There is a special call (called WINDOWS INIT BROADCAST),

which can be used to force an V86 manager to switch to Real mode (of course,

the V86 manager must support it). This call was originally used by

MS Windows while starting, while an Expanded Memory Manager was running

(this is needed, because EMM is running in Protected mode and Windows too).

Such known EMM managers are:

- Microsoft EMM386 Expanded Memory Manager

- Quarterdeck QEMM386 Expanded Memory Manager

- Qualitas 386MAX Expanded Memory Manager

- Nu-Mega Soft-ICE Debugger

...

So HWiNFO uses this call to force the manager to switch back to Real mode.

HWiNFO works best under EMM386 (I haven't found any problems).

But there are some disadvantages while runnig in V86 mode:

- The FPU benchmark returns another results (it's slower) as in real mode.

- Some V86 managers are able to relocate BIOSes, so HWiNFO won't be able

to identify BIOS type and some video cards.

- A crash while detecting the CPU type could happen on some older

CPUs requiring CPU RESET to identify them properly.

The other case is, if a manager doesn't support the call. This happens if

you are running under Microsoft Windows95 OSR2 or Windows98 DOS mode.

If these managers receive the Windows INIT BCAST, they display a message

about running an incompatible application and stop. So the only way is

to skip the privileged routines.

Pre v4.4.1 of HWiNFO refused to run under this circumstances, but

later versions run with the following restrictions (skip privileged routs):

- CPU frequency can't be measured using TSC

- CPU RESET can't be performed

(this is needed only for older CPUs or to receive the Processor Number

while this feature has been disabled)

- TI486 Potomac CPUs can't be identified

- L2 cache ECC presence can't be detected (P-II, Xeon)

- CPU Update Revision can't be reported (PPro and above)

- Cache state, branch prediction and instruction execution can't be reported

If you try to run HWiNFO directly from Microsoft Windows (DOS box), it will

refuse to continue. This is because Windows controls also access to some

ports and doesn't allow a direct access from a DOS application. This

means, that if HWiNFO wouldn't refuse to run, but continue, it would be

unable to return info about IDE/ATAPI drives, COM/LPT ports, BIOS scan,

PnP resources, Benchmarks wouldn't be accurate...

Windows NT VDM (Virtual DOS Machine) is even more strict.

## 9. Resetting the CPU ######################################################

The CPU ID is available only on 386+ CPUs and HWiNFO uses three ways to

obtain this ID:

Method 1 - Resets the CPU while A20 is Enabled.

This method works on all systems, but the Reset handler gets

control after BIOS. And some BIOSes (some newer AMI/Award BIOSes)

destroy CPU ID (EDX) and therefore this ID could be invalid.

This method is default <Enabled> in Setup section.

Method 2 - Resets the CPU while A20 is Disabled.

This method works only on some machines, which do have a

Fully-Terminated bus. It's the best way to obtain CPU ID, because

it gets control immediately after Reset (the BIOS can't destroy

EDX). I've found some problems with this method while running

with Norton cache.

This method is default <Disabled> in Setup section.

Method 3 - Try to use this method if your BIOS destroys CPU ID. But it works

only on systems, which do not have Main BIOS (area at 0F000:0)

Write-Protected, because it modifies it for a bit, and then

restores back. This method is on some machines the only way

to obtain CPU ID.

This method is default <Enabled> in Setup section.

HWiNFO tries to determine CPU ID this way, via: 1. CPUID instruction

2. BIOS (INT 15h)

3. Method 2 (if Enabled)

4. Method 3

5. Method 1

If you have an UMC U5SX/SD/SLV/486DX2/SX2, Enhanced Am486DX2/DX4/DX4-WB,

AMD-K5 and above, Intel 486DX/SX/DX2 SL-Enhanced, IntelDX4, IntelSX2,

IntelDX2 W-B Enh. or Intel Pentium(R) and above, there is no need to Reset

the CPU (these CPU support CPUID instruction). Most CPUs today support

CPUID instruction.

Note: NexGen Nx586 does NOT support the CPUID instruction.

Cyrix 5x86 and above support the CPUID instruction only if CPUIDEN bit

in CCR4 register is set. HWiNFO 3.0.7+ sets this bit automatically.

Disabling "CPU ID detection via Method 1, 2 and 3" reduces your CPU

detection ability. And if there's no other way to obtain this ID

(via BIOS or CPUID instruction), it won't identify 386SL, i386CX/EX,

486DX2, IBM 486... and some other CPUs. Do this only if your system

hangs while attempting CPU detection.

Some BIOSes simply OVERWRITE the CPU ID (EDX) :-< ! In this case an

invalid CPU ID is displayed and the CPU is probably incorrectly identified.

If this problem occurs, try to use Method 3 (but your Main BIOS must be

Write-Enabled !). If it doesn't work, try to use Method 2 (but you must

have a Fully-Terminated bus !). And if HWiNFO still displays, that Method 1

was used to obtain CPU ID, not Method 2 nor 3 can obtain it.

## 10. HWiNFO Crash ##########################################################

- If HWiNFO crashes while executing the CPU benchmark, please use

HIMEM.SYS (or another XMS manager).

- S3 ViRGE cards seem to crash while performing the TV Tuner test.

If so, disable the "Check TV Tuner" option in Setup.

- Stacker Compression Card and 3Com EtherLink III Adapter detection

routines can cause crashes with some Network cards. You can disable

them in the Configuration section (Stacker check is default disabled;

3Com EtherLink III detection is enabled).

- If HWiNFO crashes while starting or while the "Flushing Disk Caches..."

message is displayed, disable the "FlushDiskCache" option in HWiNFO.INI

or HWiNFO|Setup.

- The test for a TIGA graphics adapter may cause problems with some

adapters.

## 11. Secondary Video Info ##################################################

RISC Graphics Processor: currently identifies only Hitachi HD63484 ACRTC

Scanning for TMS: checks for TMS340x0 processor on several ports and tries

to get vendor and device name (detection routine from

SPEA GDC drivers, I don't know if it works with other TMS

cards too)

TMS Boot EEPROM: gets info from Boot Software; this won't probably work

with other cards than SPEA (I don't know :-( )

TIGA driver: gets info from TIGA driver if it's loaded (TIGACD.EXE)

Also found as: identifies all SPEA Graphiti series cards from GDCINI.DAT

and checks for HP IGC-20 and NEC MultiSync Graphics Engine

You can obtain info such as:

e.g.: RISC Graphics Processor:   Hitachi HD63484 (ACRTC)

Scanning for TMS:          N/A

TMS Boot EEPROM:           N/A

TIGA driver:               N/A

Also found as:             SPEA Graphiti Flash 1G2-HE

or:   RISC Graphics Processor:   N/A

Scanning for TMS:          (C) SPEA Software AG HILITE1024

TMS Boot EEPROM:           SPEA HiLite1024

TIGA driver:               N/A

Also found as:             SPEA Graphiti HiLite1024

or:   RISC Graphics Processor:   N/A

Scanning for TMS:          <I really don't know what it'll display>

TMS Boot EEPROM:                             -||-

TIGA driver:               HP A1083A Intelligent Graphics Controller 20

Also found as:             Hewlett Packard IGC-20 A1083A

## 12. PCI chipsets comparison ###############################################

������������������������������������������������������������������������Ŀ

� Chipset        � Intel � Intel � Intel � Intel � Intel � Intel � Intel �

�                � 430LX � 430NX � 430FX � 430VX � 430HX � 430MX � 430TX �

������������������������������������������������������������������������Ĵ

� North Bridge   �82434LX�82434NX�82437FX�82437VX�82439HX�82437MX�82379TX�

�                � PCMC  � PCMC  �  TSC  �  TVX  �  TXC  � MTSC  � MTXC  �

������������������������������������������������������������������������Ĵ

� Supported      �  P5   �  P5,  � P54C  � P54C  � P54C  � P54LM � P54C, �

� CPUs           �       �  P54C � K5,   � K5,   � K5,   �mobile � P55C, �

�                �       �       � M1    � M1    � M1    �       � K5,K6 �

�                �       �       �       �       �       �       � M1,M2 �

������������������������������������������������������������������������Ĵ

� Max. bus freq. �  66   �  66   �  66   �  66   �  66   �  66   �  66   �

������������������������������������������������������������������������Ĵ

� SMP (max CPUs) �   -   �   -   �   -   �   -   �   2   �   -   �   -   �

������������������������������������������������������������������������Ĵ

� Max. memory    � 192M  � 512M  � 128M  � 128M  � 512M  � 128M  � 256M  �

������������������������������������������������������������������������Ĵ

� Supported      �  SPM  �  SPM  �  SPM, �  FPM, �  FPM, �  SPM, �  FPM, �

� Memory         �       �       �  EDO  �  EDO, �  EDO  �  EDO  �  EDO, �

� Types          �       �       �       � SDRAM �       �       � SDRAM �

������������������������������������������������������������������������Ĵ

� ECC/Parity     �   -   �   -   �   -   �   -   �   X   �   -   �   -   �

������������������������������������������������������������������������Ĵ

� Cacheability   � 192M  � 512M  �  64M  �  64M  �  512M �  64M  �  64M  �

������������������������������������������������������������������������Ĵ

� South Bridge   �82378IB�82378IB�82371FB�82371SB�82371SB�82371MX�82371AB�

�                �  SIO  �  SIO  � PIIX  � PIIX3 � PIIX3 � MPIIX � PIIX4 �

������������������������������������������������������������������������Ĵ

� PCI support    �  2.0  �  2.0  �  2.0  �  2.1  �  2.1  �  2.0  �  2.1  �

������������������������������������������������������������������������Ĵ

� A.G.P.         �   -   �   -   �   -   �   -   �   -   �   -   �   -   �

������������������������������������������������������������������������Ĵ

� Concurrent PCI �   -   �   -   �   -   �   X   �   X   �   -   �   X   �

������������������������������������������������������������������������Ĵ

� USB            �   -   �   -   �   -   �  1.0  �  1.0  �   -   �  1.0  �

������������������������������������������������������������������������Ĵ

� Power Mgmt.    �   -   �   -   �   -   �   -   �   -   �SMI/APM� ACPI  �

������������������������������������������������������������������������Ĵ

� Ultra-DMA mode �   -   �   -   �   -   �   -   �   -   �   -   �   2   �

��������������������������������������������������������������������������

������������������������������������������������������������������������������������������������Ŀ

� Chipset        � Intel � Intel � Intel � Intel � Intel � Intel � Intel � Intel � Intel � Intel �

�                � 440FX � 450GX � 450KX � 440LX � 440EX � 440BX � 440ZX � 440MX � 440GX � 450NX �

������������������������������������������������������������������������������������������������Ĵ

� North Bridge   �82441FX�82453GX�82453KX�82443LX�82443EX�82443BX�82443ZX�82443MX�82443GX�82451NX�

�                �  PMC  �82454GX�82454KX�  PAC  �  PAC  �       �       �       �       �82454NX�

������������������������������������������������������������������������������������������������Ĵ

� Supported      � P6,   � P6    � P6    � P-II  � P-II, � P-II  � P-II, �Mobile � P-II, � Xeon  �

� CPUs           � P-II  �       �       �       �Celeron�       �Celeron� P-II/ � Xeon  �       �

�                �       �       �       �       �       �       �optimi-� P-III,�       �       �

�                �       �       �       �       �       �       �  zed  �Celeron�       �       �

������������������������������������������������������������������������������������������������Ĵ

� Max. bus freq. �  66   �  66   �  66   �  66   �  66   �  100  �  100  �  66   �  100  �  100  �

������������������������������������������������������������������������������������������������Ĵ

� SMP (max CPUs) �   2   �   4   �   2   �   2   �   -   �   2   �   -   �   -   �   2   �   4   �

������������������������������������������������������������������������������������������������Ĵ

� Max. memory    �  1G   �  8G   �  1G   �1G(EDO)� 256M  �  1G   � 256M  � 256M  �  2G   �  8G   �

������������������������������������������������������������������������������������������������Ĵ

� Supported      �FPM,EDO�  FPM  �  FPM  � EDO,  � EDO,  � EDO,  � SDRAM � SDRAM � SDRAM �  EDO  �

� Memory         � BEDO  �       �       � SDRAM � SDRAM � SDRAM �       �       �       �       �

� Types          �       �       �       �       �       �   **  �       �       �       �       �

������������������������������������������������������������������������������������������������Ĵ

� ECC/Parity     �   X   �   X   �   X   �   X   �   -   �   X   �   -   �   -   �   X   �   X   �

������������������������������������������������������������������������������������������������Ĵ

� Cacheability   �  ***  �  ***  �  ***  �  ***  �  ***  �  ***  �  ***  �  ***  �  ***  �  ***  �

������������������������������������������������������������������������������������������������Ĵ

� South Bridge   �82371SB�optio- �optio- �82371AB�82371AB�82371EB�82371EB�integr-�82371EB�82371EB�

�                � PIIX3 � nal   � nal   � PIIX4 � PIIX4 �PIIX4E �PIIX4E � ated  �PIIX4E �PIIX4E �

������������������������������������������������������������������������������������������������Ĵ

� PCI support    �  2.1  �  2.0  �  2.0  �  2.1  �  2.1  �  2.1  �  2.1  �  2.2  �  2.1  �  2.1  �

������������������������������������������������������������������������������������������������Ĵ

� A.G.P.         �   -   �   -   �   -   �  2x   �  2x   �  2x   �  2x   �   -   �  2x   �   -   �

������������������������������������������������������������������������������������������������Ĵ

� Concurrent PCI �   X   � ****  � ****  �   X   �   X   �   X   �   X   �   X   �   X   �   X   �

������������������������������������������������������������������������������������������������Ĵ

� USB            �  1.0  � ****  � ****  �  1.0  �  1.0  �  1.0  �  1.0  �  1.0  �  1.0  �  1.0  �

������������������������������������������������������������������������������������������������Ĵ

� Power Mgmt.    �  SMM  �  SMM  �  SMM  � ACPI  � ACPI  � ACPI  � ACPI  � ACPI  � ACPI  � ACPI  �

������������������������������������������������������������������������������������������������Ĵ

� Ultra-DMA mode �   -   �   -   �   -   �   2   �   2   �   2   �   2   �   2   �   2   �   2   �

��������������������������������������������������������������������������������������������������

��������������������������������������������������������������������������Ŀ

� Chipset        � Intel � Intel � Intel � Intel � Intel �  Intel/ � Intel �

�                � i810  � i810E � i820  � i840  � i815  �Corollary� i850  �

�                �Whitney�       �Camino �Carmel �Solano �PROfusion�       �

��������������������������������������������������������������������������Ĵ

� North Bridge / � 82810 � 82810E� 82820 � 82840 � 82815 �         � 82850 �

� Hub            � GMCH  �  GMCH �       �       �       �         �       �

��������������������������������������������������������������������������Ĵ

� Supported      �Celeron�Celeron� P-II, � P-III � P-III �  P-III  �Pentium�

� CPUs           � P-II, � P-II, � P-III �       �       �  Xeon   �   4   �

�                � P-III � P-III �       �       �       �         �       �

�                �       �       �       �       �       �         �       �

��������������������������������������������������������������������������Ĵ

� Max. bus freq. �  100  �  133  �  133  �  133  �  133  �   100   �400 QDR�

��������������������������������������������������������������������������Ĵ

� SMP (max CPUs) �   -   �   -   �   2   �   4   �   -   �    8    �   -   �

��������������������������������������������������������������������������Ĵ

� Max. memory    �  256M �  256M �  1G   �  2G   � 512M  �   32G   �       �

��������������������������������������������������������������������������Ĵ

� Supported      � SDRAM � SDRAM � RDRAM,� SDRAM,� PC133 �  SDRAM  � RDRAM �

� Memory         �       �       � SDRAM �2-chnl � SDRAM �         �       �

� Types          �       �       � w/MTH � RDRAM �       �         �       �

��������������������������������������������������������������������������Ĵ

� ECC/Parity     �   -   �   -   �   X   �   X   �   -   �    X    �       �

��������������������������������������������������������������������������Ĵ

� South Bridge / �82801Ax�82801Ax�82801Ax�82801AA�82801xA�         �82801BA�

� Hub            � ICH/0 � ICH/0 � ICH/0 �  ICH  � ICH/2 �         �  ICH2 �

��������������������������������������������������������������������������Ĵ

� PCI support    �  2.2  �  2.2  �  2.2  � ***** �  2.2  � 66 MHz  �  2.2  �

��������������������������������������������������������������������������Ĵ

� A.G.P.         � (2x)  � (2x)  �  4x   �  4x   �  4x   �         �  4x   �

��������������������������������������������������������������������������Ĵ

� USB            �  1.0  �  1.0  �  1.0  �  1.0  �  1.0  �   1.0   �  1.0  �

��������������������������������������������������������������������������Ĵ

� Power Mgmt.    � ACPI  � ACPI  � ACPI  � ACPI  � ACPI  �  ACPI   � ACPI  �

��������������������������������������������������������������������������Ĵ

� Ultra-DMA mode �  2/4  �   4   �   4   �   4   �   4   �         �   5   �

��������������������������������������������������������������������������Ĵ

� Graphics       �   X   �   X   �   -   �   -   �  *1   �    -    �   -   �

��������������������������������������������������������������������������Ĵ

� Audio          � AC'97 � AC'97 � AC'97 � AC'97 � AC'97 �    -    �       �

����������������������������������������������������������������������������

***** - Supports PCI 2.2; and PCI64 (64-bit, 66 MHz)

with optional 802806AA PCI64 Hub

*1    - Intergrated graphics adapter, that can be disabled

i820E and i815E chipsets use i820/i815 North Bridge, but 80801BA ICH2

with ATA-100 and 2xUSB (4-port) support

i815EP is i815E shipped without integrated graphics controller

i815EM is i815 with mobile support

������������������������������������������������������������������������������������������������Ŀ

�                �  VIA  �  VIA  �  VIA  �  VIA  �  VIA  �  VIA  �  VIA  �  VIA  �  VIA  �  VIA  �

� Chipset        �Apollo �Apollo �Apollo �Apollo �Apollo �Apollo �Apollo �Apollo �Apollo �Apollo �

�                �  VP1  �  VPX  �  VP2  �  VP3  �  MVP3 � MVP4  �  Pro  �ProPlus�Pro133 �Pro133A�

������������������������������������������������������������������������������������������������Ĵ

� North Bridge   �82C585 �82C585 �82C595 �82C597 �82C598 � 8501  �82C691 �82C693 �82C693A�82C694X�

�                �  VP   �  VPX  �       �  /AT  �  /AT  �       �       �       �       �       �

������������������������������������������������������������������������������������������������Ĵ

� Supported      � P54C, � P54C, � P54C, � P54C, � P54C, � P54C, � P6,   � P-II, � P-II, � P-II, �

� CPUs           � K5,   � P55C, � P55C, � P55C, � P55C, � P55C, � P-II  �Celeron� P-III,� P-III �

�                � M1    � K5,K6,� K5,K6,� K5,K6,� K5,K6,� K5,K6,�       �       �Celeron�       �

�                �       � M1,M2 � M1,M2 � M1,M2 � M1,M2 � M1,M2 �       �       �       �       �

������������������������������������������������������������������������������������������������Ĵ

� Max. bus freq. �  66   �  75   �  66   �  100  �  100  �  100  �  100  �  100  �  133  �  133  �

������������������������������������������������������������������������������������������������Ĵ

� SMP (max CPUs) �   -   �   -   �   -   �   -   �   -   �   -   �   -   �   -   �   -   �   -   �

������������������������������������������������������������������������������������������������Ĵ

� Max. memory    � 512M  � 512M  � 768M  �  1G   �  1G   � 768M  �  1G   �  1G   �  1.5G �       �

������������������������������������������������������������������������������������������������Ĵ

� Supported      � FPM,  � FPM,  � FPM,  �FPM,EDO�FPM,EDO�FPM,EDO�FPM,EDO�FPM,EDO�FPM,EDO�SDRAM, �

� Memory         � EDO,  � EDO,  � EDO,  �SDRAM, �SDRAM, �SDRAM, �SDRAM, �SDRAM, �SDRAM, � DDR   �

� Types          � SDRAM � SDRAM � SDRAM �SDRAM2 �SDRAM2 �VC DRAM�SDRAM2 �  VCM  �  VCM  �       �

������������������������������������������������������������������������������������������������Ĵ

� ECC/Parity     �   -   �   -   �   X   �   X   �   X   �   X   �   X   �   X   �   X   �       �

������������������������������������������������������������������������������������������������Ĵ

� Cacheability   � 512M* � 512M* � 512M* � 512M *� 512M *� 512M *�  ***  �  ***  �  ***  �  ***  �

������������������������������������������������������������������������������������������������Ĵ

� South Bridge   �82C586 �82C586B�82C586B�82C586B�82C586B�82C686A�82C596A�82C596A�82C596B�82C596B�

�                �       �       �       �       �       �       �       �       �       �       �

������������������������������������������������������������������������������������������������Ĵ

� PCI support    �  2.1  �  2.1  �  2.1  �  2.1  �  2.1  �  2.2  �  2.1  �  2.1  �  2.2  �  2.2  �

������������������������������������������������������������������������������������������������Ĵ

� A.G.P.         �   -   �   -   �   -   �  2x   �  2x   �  2x   �  2x   �  2x   �  2x   �  4x   �

������������������������������������������������������������������������������������������������Ĵ

� USB            �   X   �   X   �   X   �   X   �   X   �   X   �   X   �   X   �   X   �   X   �

������������������������������������������������������������������������������������������������Ĵ

� Power Mgmt.    �  APM  �  ACPI �  ACPI � ACPI  � ACPI  � ACPI  � ACPI  � ACPI  � ACPI  � ACPI  �

������������������������������������������������������������������������������������������������Ĵ

� Ultra-DMA mode �   -   �   2   �   2   �   2   �   2   �   4   �   4   �   4   �   4   �   4   �

������������������������������������������������������������������������������������������������Ĵ

� Misc           �       �       �       �       � Mobile� VGA,  �       �       � APIC  �       �

�                �       �       �       �       �       � Audio,�       �       �       �       �

�                �       �       �       �       �       � Modem �       �       �       �       �

��������������������������������������������������������������������������������������������������

����������������������������������������Ŀ

�                �  VIA  �  VIA  �  VIA  �

� Chipset        � KX133 � KT133 � KM133 �

�                �       �       �       �

����������������������������������������Ĵ

� North Bridge   �VT8371 �VT8363 �VT8365 �

�                �       �       �       �

����������������������������������������Ĵ

� Supported      �Athlon �Athlon,�Athlon �

� CPUs           �       �Duron  �       �

�                �       �       �       �

�                �       �       �       �

����������������������������������������Ĵ

� Max. FSB freq. �  200  �  200  �  200  �

����������������������������������������Ĵ

� SMP (max CPUs) �   -   �   -   �   -   �

����������������������������������������Ĵ

� Max. memory    �  2G   �       �       �

����������������������������������������Ĵ

� Supported      � PC100,� PC100,� PC100,�

� Memory         � PC133,� PC133,� PC133,�

� Types          � VCM   � VCM   � VCM   �

����������������������������������������Ĵ

� ECC/Parity     �   -   �   -   �   X   �

����������������������������������������Ĵ

� South Bridge   �82C686A�82C686A�82C686A�

�                �       �       �       �

����������������������������������������Ĵ

� PCI support    �  2.2  �  2.2  �  2.2  �

����������������������������������������Ĵ

� A.G.P.         �  4x   �  4x   �  4x   �

����������������������������������������Ĵ

� USB            �   X   �   X   �   X   �

����������������������������������������Ĵ

� Power Mgmt.    �  ACPI �  ACPI �  ACPI �

����������������������������������������Ĵ

� Ultra-DMA mode �   4   �   4   �   4   �

����������������������������������������Ĵ

� Misc           �       �       �Integr.�

�                �       �       �Savage4�

�                �       �       �       �

������������������������������������������

����������������������������������������Ŀ

�                �AMD-750�AMD-760�AMD-760�

� Chipset        �       �       �   MP  �

�                �       �       �       �

����������������������������������������Ĵ

� North Bridge   �AMD-751�AMD-761�AMD-762�

�                �       �       �       �

����������������������������������������Ĵ

� Supported      �Athlon �Athlon,�Athlon,�

� CPUs           �       �Duron  �Duron  �

�                �       �       �       �

�                �       �       �       �

����������������������������������������Ĵ

� Max. FSB freq. �200 DDR�266 DDR�266 DDR�

����������������������������������������Ĵ

� SMP (max CPUs) �   -   �   -   �   Y   �

����������������������������������������Ĵ

� Max. memory    � 768M  �       �       �

����������������������������������������Ĵ

� Supported      � PC100 � DDR   � DDR   �

� Memory         � SDRAM � SDRAM � SDRAM �

� Types          �       �       �       �

����������������������������������������Ĵ

� ECC/Parity     �   X   �   X   �   X   �

����������������������������������������Ĵ

� South Bridge   �AMD-756�AMD-766�AMD-766�

�                �       �       �       �

����������������������������������������Ĵ

� PCI support    �  2.2  �  2.2  �  2.2  �

����������������������������������������Ĵ

� A.G.P.         �  2x   �  4x   �  4x   �

����������������������������������������Ĵ

� USB            �  1.0  �  1.0  �  1.0  �

����������������������������������������Ĵ

� Power Mgmt.    �  ACPI �  ACPI �  ACPI �

����������������������������������������Ĵ

� Ultra-DMA mode �   4   �   5   �   5   �

����������������������������������������Ĵ

� Misc           �       �       �       �

�                �       �       �       �

�                �       �       �       �

������������������������������������������

������������������������������������������������������������������������������������������������Ŀ

� Chipset        � SiS   �  SiS  � SiS   � SiS   � SiS   � SiS   �  SiS  �  SiS  �  SiS  �  SiS  �

�                � 5511  �  5596 � 5571  � 5581  � 5597  � 5591  �  530  �  600  �  620  �  630  �

������������������������������������������������������������������������������������������������Ĵ

� North Bridge   � 5511  � 5596  � 5571  � 5581  � 5597  � 5591  �  530  �  600  �85C620 �85C630 �

�                � PCMC  �       �       �       �       �       �       �       �       �       �

������������������������������������������������������������������������������������������������Ĵ

� Supported      � P54C, � P54C, � P54C, � P54C, � P54C, � P54C, � P54C, � P-II  � P-II  � P-II, �

� CPUs           � K5,   � K5,   � P55C, � P55C, � P55C, � P55C, � P55C, �       �       � P-III �

�                � M1    � M1    � K5,K6,� K5,K6,� K5,K6,� K5,K6,� K5,K6 �       �       �       �

�                �       �       � M1,M2 � M1,M2 � M1,M2 � M1,M2 � M1,M2 �       �       �       �

������������������������������������������������������������������������������������������������Ĵ

� Max. bus freq. �  66   �  66   �  75   �  75   �  75   �  66   �  100  �  100  �  100  �  100  �

������������������������������������������������������������������������������������������������Ĵ

� SMP (max CPUs) �   -   �   -   �   -   �   -   �   -   �   -   �   -   �   -   �   -   �   -   �

������������������������������������������������������������������������������������������������Ĵ

� Max. memory    � 512M  � 512M  � 384M  � 384M  � 384M  � 768M  � 1.5G  �  1.5G �  1.5G �  1.5G �

������������������������������������������������������������������������������������������������Ĵ

� Supported      � FPM,  � FPM,  � FPM,  � FPM,  � FPM,  � FPM,  � SDRAM � FPM,  � SDRAM � SDRAM �

� Memory         � EDO   � EDO   � EDO,  � EDO,  � EDO,  � EDO,  �       � EDO,  �       �       �

� Types          �       �       � SDRAM � SDRAM � SDRAM � SDRAM �       � SDRAM �       �       �

������������������������������������������������������������������������������������������������Ĵ

� ECC/Parity     �   -   �   -   �   -   �   -   �   -   �   X   �   -   �   X   �   -   �   -   �

������������������������������������������������������������������������������������������������Ĵ

� Cacheability   � 256M *� 256M *� 128M *� 128M *� 128M *� 256M *� 256M *�  ***  �  ***  �  ***  �

������������������������������������������������������������������������������������������������Ĵ

� South Bridge   � 5513  � 5513  � 5513  � 5513  � 5513  � 5595  � 5595  � 5595  � 5595  �integr.�

�                �       �       �integr.�integr.�integr.�       �       �       �       � 5595  �

������������������������������������������������������������������������������������������������Ĵ

� PCI support    �  2.0  �  2.0  �  2.1  �  2.1  �  2.1  �  2.1  �  2.2  �  2.1  �  2.2  �  2.2  �

������������������������������������������������������������������������������������������������Ĵ

� A.G.P.         �   -   �   -   �   -   �   -   �   -   �  2x   �  2x   �  2x   �  2x   �  4x   �

������������������������������������������������������������������������������������������������Ĵ

� Concurrent PCI �   X   �   X   �   X   �   X   �   X   �   X   �   X   �   X   �   X   �   X   �

������������������������������������������������������������������������������������������������Ĵ

� USB            �   -   �   -   �   X   �   X   �   X   �   X   �   X   �   X   �   X   �  2x   �

������������������������������������������������������������������������������������������������Ĵ

� Power Mgmt.    �  SMM  �  SMM  �  SMM  � ACPI  � ACPI  � ACPI  � ACPI  � ACPI  � ACPI  � ACPI  �

������������������������������������������������������������������������������������������������Ĵ

� Ultra-DMA mode �   -   �   -   �   -   �   2   �   2   �   2   �   4   �   2   �   4   �   4   �

��������������������������������������������������������������������������������������������������

��������������������������������������������������������Ŀ

� Chipset        �  ALi  �  ALi  �  ALi  �  ALi  �  ALi  �

�                �Aladdin�Aladdin�Aladdin�Aladdin�Aladdin�

�                �  III  �  IV   �  IV+  �   V   �   7   �

��������������������������������������������������������Ĵ

� North Bridge / � M1521 � M1531 � M1531 � M1541 � M1561 �

� Hub            �       �       �       �       �       �

��������������������������������������������������������Ĵ

� Supported      � P54C, � P54C  � P54C, � P54C, � P54C, �

� CPUs           � P55C, � P55C, � P55C, �P55C,K5�P55C,K6�

�                � K5,K6,� K5,K6,� K5,K6,�K6,K6-2� K6-2, �

�                � M1,M2 � M1,M2 � M1,M2 � M1,M2 � K6-III�

��������������������������������������������������������Ĵ

� Max. bus freq. �  66   �  83   �  83   �  100  �  100  �

��������������������������������������������������������Ĵ

� SMP (max CPUs) �   -   �   -   �   -   �   -   �   -   �

��������������������������������������������������������Ĵ

� Max. memory    �  1G   �  1G   �  1G   �  1G   �  1G   �

��������������������������������������������������������Ĵ

� Supported      � EDO,  � EDO,  � EDO,  � EDO,  � PC66, �

� Memory         � SDRAM � SDRAM � SDRAM � SDRAM � PC100 �

� Types          �       �       �       �       � SDRAM �

��������������������������������������������������������Ĵ

� ECC/Parity     �   X   �   X   �   X   �   X   �   X   �

��������������������������������������������������������Ĵ

� South Bridge   � M1523 � M1533 � M1543 � M1543C� M1543 �

��������������������������������������������������������Ĵ

� PCI support    �  2.1  �  2.1  �  2.1  �  2.1  �  2.2  �

��������������������������������������������������������Ĵ

� A.G.P.         �   -   �   -   �   -   �  2x   �   -   �

��������������������������������������������������������Ĵ

� USB ports      �   2   �   2   �   2   �   3   �   4   �

��������������������������������������������������������Ĵ

� Power Mgmt.    �  APM  � ACPI  � ACPI  � ACPI  � ACPI  �

��������������������������������������������������������Ĵ

� Ultra-DMA mode �   -   �   2   �   2   �   2   �   4   �

��������������������������������������������������������Ĵ

� Graphics       �   -   �   -   �   -   �   -   � ArtX  �

��������������������������������������������������������Ĵ

� Audio          �   -   �   -   �   -   �   -   �   -   �

����������������������������������������������������������

������������������������������������������������������������������������Ŀ

� Chipset        �  ALi  �  ALi  �  ALi  �  ALi  �  ALi  �  ALi  �  ALi  �

�                �Aladdin�Aladdin�Aladdin�CyblerB�Aladdin�Aladdin� MAGiK1�

�                � Pro I �Pro II �Pro III�Alad.i1�Pro IV � Pro 5 �       �

������������������������������������������������������������������������Ĵ

� North Bridge / � M1615 � M1621 � M1631 � M1632M� M1641/� M1651 � M1647 �

� Hub            �       �       �       �       � M1641B�       �       �

������������������������������������������������������������������������Ĵ

� Supported      �  P6   �  P-II � P-II, � Mobile� P-II, � P-II  �Athlon,�

� CPUs           �       �       � P-III,�       � P-III,� P-III,� Duron �

�                �       �       �Celeron�       �Celeron�Celeron�       �

�                �       �       �       �       �       �       �       �

������������������������������������������������������������������������Ĵ

� Max. bus freq. �  66   �  100  �  133  �  100  �100/133�  133  �266 DDR�

������������������������������������������������������������������������Ĵ

� SMP (max CPUs) �   -   �   -   �   -   �   -   �   -   �   -   �   -   �

������������������������������������������������������������������������Ĵ

� Max. memory    �  2G   �  2G   � 1.5G  �       � 1.5G  �  3G   �  3G   �

������������������������������������������������������������������������Ĵ

� Supported      � FPM,  � FPM,  � EDO,  � SDRAM � EDO,SD� PC100,�PC133, �

� Memory         � EDO,  � EDO,  � SDRAM,�       � /DDR  � PC133,�PC1600,�

� Types          � SDRAM � SDRAM � VCM   �       �       � DDR   �PC2100 �

������������������������������������������������������������������������Ĵ

� ECC/Parity     �   X   �   X   �   X   �   -   �   X   �   X   �   -   �

������������������������������������������������������������������������Ĵ

� South Bridge   � M1533 � M1543C� M1543D� M1535+� M1535D�M1535D+�M1535D+�

������������������������������������������������������������������������Ĵ

� PCI support    �  2.1  �  2.1  �  2.2  �  2.2  �  2.2  �  2.2  �  2.2  �

������������������������������������������������������������������������Ĵ

� A.G.P.         �   -   �  2x   �  2x   �  2x   �  4x   �  4x   �  4x   �

������������������������������������������������������������������������Ĵ

� USB ports      �   2   �   2   �   4   �   4   �   4   �   4   �   4   �

������������������������������������������������������������������������Ĵ

� Power Mgmt.    � ACPI  � ACPI  � ACPI  � ACPI  � ACPI  � ACPI  � ACPI  �

������������������������������������������������������������������������Ĵ

� Ultra-DMA mode �   2   �   2   �   4   �   4   �   4   �   5   �   5   �

������������������������������������������������������������������������Ĵ

� Graphics       �   -   �   -   �nV TNT2�Trident�   -   �   -   �   -   �

������������������������������������������������������������������������Ĵ

� Audio          �   -   �   -   �   -   �   -   �   -   � AC'97 � AC'97 �

��������������������������������������������������������������������������

*    - Depends on L2 cache size:

�����������������������Ŀ

� cache �   cacheable   �

� size  �    region     �

�����������������������Ĵ

� 256K  �64M-cache_size �

� 512K  �128M-cache_size�

� 1M    �256M-cache_size�

� 2M    �512M-cache_size�

�������������������������

Some chipsets (SiS) may cache only the half of the given memory size.

**   - EDO and SDRAM cannot be mixed

***  - no L2 cache on-board (depends on on-chip L2 cache size in CPU)

**** - available if 82371SB PIIX3 is used

## 13. Hardware monitor chip support #########################################

Sensor chip              Temps  Voltages  FAN  Interface      Misc

-------------------------------------------------------------------------------

National Semi. LM75        1       -       -     SMBus

Genesys GL523SM            1       -       -     SMBus

Maxim MX1617               2       -       -     SMBus

Maxim MX1617A              2       -       -     SMBus

Analog Devices ADM1021     2       -       -     SMBus

Analog Devices ADM1022     3       -       -     SMBus

TI THMC50                  1       -       -     SMBus

Fairchild FMS2701          1       -       -     SMBus

National Semi. LM78        1       6       3     ISA, SMBus

National Semi. LM78-J      1       6       3     ISA, SMBus

National Semi. LM79        1       6       3     ISA, SMBus

National Semi. LM80        -       6       2     SMBus

Winbond W83781D            3       7       3     ISA, SMBus

Winbond W83782D            3       9       3     ISA, SMBus   5VSB, VBAT voltages

Winbond W83783S            2       6       3     SMBus

Winbond W83L784LR          3       4       2     SMBus        Battery voltage

SMSC MON35W82              3       9       3     ISA, SMBus   5VSB, VBAT voltages

SMSC LPC61W492             3       9       3     ISA, SMBus   5VSB, VBAT voltages

Analog Devices ADM9240     1       6       2     SMBus

National Semi. LM81        1       6       2     SMBus

Dallas Semi. DS1780        1       6       2     SMBus

Analog Devices ADM1024     2       6       2     SMBus

Analog Devices ADM1025     2       6       2     SMBus

National Semi. LM87        2       6       2     SMBus

Analog Devices ADM1026     3      16       8     SMBus

Analog Devices ADM1028     2       -       1     SMBus

Analog Devices ADM1029     2       -       2     SMBus

Analog Devices ADM1030     2       -       1     SMBus

VIA VT82C686 Internal      3       7       2     ISA

SiS 5595                   1       4       2     ISA

SiS 950 LPC                3       8       3     SMBus

Genesys GL518SM            1       1       2     SMBus

Genesys GL520SM            2       4       2     SMBus

Genesys GL525SM            2       5       3     SMBus

-------------------------------------------------------------------------------

Detection of chips with SMBus interface requires an Intel 82371AB/EB PIIX4,

82801AA/AB ICH/ICH0, 82801BA ICH2, ALI M7101, VIA 586B, 596A/B or 686A

SMBus Host Controller.

Intel Mobile Pentium II, Pentium II Mobile Module, Pentium II Xeon and

Pentium III Xeon Processors use the Maxim MX1617 (or compatible) integrated

temperature sensor.

## 14. IDE Disk Transfer Modes ###############################################

HWiNFO reports transfer modes for all IDE/ATAPI drives, which support it.

Max. PIO transfer mode:   PIO x

DMA Transfer Modes:       SW: a1 (m1), MW: a2 (m2), Ultra: a3 (m3)

Where:

x = Maximum supported PIO transfer mode

SW = Singleword DMA transfer mode

MW = Multiword DMA transfer mode

Ultra = Ultra-DMA transfer mode

a1, a2, a3 = Current active mode

m1, m2, m3 = Max. supported mode

�����������������������������Ŀ

� Transfer    � Maximum       �

� Mode        � Transfer rate �

�����������������������������Ĵ

� PIO 0       �    3.33 MB/s  �

� PIO 1       �    5.22 MB/s  �

� PIO 2       �    8.33 MB/s  �

� PIO 3       �   11.11 MB/s  �

� PIO 4       �   16.66 MB/s  �

�����������������������������Ĵ

� DMA 0       �    4.16 MB/s  �

� DMA 1       �   13.33 MB/s  �

� DMA 2       �   16.66 MB/s  �

�����������������������������Ĵ

� Ultra-DMA 0 �   16.66 MB/s  �

� Ultra-DMA 1 �   25.00 MB/s  �

� Ultra-DMA 2 �   33.33 MB/s  � Ultra-DMA/33

� Ultra-DMA 3 �   44.44 MB/s  � Ultra-DMA/44

� Ultra-DMA 4 �   66.66 MB/s  � Ultra-DMA/66

� Ultra-DMA 5 �  100.00 MB/s  � ATA-100

�������������������������������

## 15. Miscellaneous notes ###################################################

On some old motherboards, random numbers are obtained from unused I/O ports

and this may cause invalid identification of some hardware (e.g. WSS).

I don't know how to solve this problem.

Intel Pentium(R) and above CPUs do not report, that the "APIC on-chip"

feature is present. Most of them (P54C yes, but P54CQS and P54LM don't)

have an on-chip APIC, but don't report it, if it isn't supported by

external hardware.

Dhrystone benchmark tests the performance of CPU and Memory, but only in

16-bit code. Therefore a 150 MHz Pentium Pro (which is designed and tuned

especially for 32-bit code) returns a rather low Dhrystone result.

So its performance while running with 16-bit applications is comparable

to a 100 MHz Pentium P54C.

If you have an Intel 420LX/420NX/430FX/430HX/430VX/430MX/430TX,

SiS 85C501/5120/5511/5571/5581/5596/5597/5591, AMD-640 chipset or

VIA Apollo VP-1/VPX/VP-2/VP3/MVP3, HWiNFO is able to detect the type of

External Cache and its size direct from the PCI chipset registers.

## 16. Some abbreviations ####################################################

FPM           = Fast-Page Mode (old standard DRAM type)

EDO           = Extended Data-Out

SDRAM         = Synchronous DRAM

DDR SDRAM II  = Double Data Rate SDRAM II

BDDR SDRAM II = Bi-directionally strobed DDR SDRAM II

BEDO          = Burst EDO

EC            = Error Check only

ECC           = Error Check and Correction

DRAM          = Dynamic RAM

VRAM          = Video RAM

WRAM          = Window RAM

SDRAM         = Synchronous DRAM

SGRAM         = Synchronous Graphics RAM

RDRAM         = Rambus DRAM

MDRAM         = Multibank DRAM

VRT           = Voltage Reduction Technology

APIC          = Advanced Programmable Interrupt Controller (required for DP)

DP            = Dual Processing

FRC           = Functional Redundancy Checking (requires DP)

MMX           = Multimedia Extensions

(57 new instructions, SIMD architecture, 64-bit regs)

MMX-2         = Some enhancements to MMX (Intel)

3DNow!        = Additional floating point MMX instructions (AMD, Cyrix)

ISSE / KNI    = Internet Streaming SIMD Extensions (Katmai New Instructions)

(70 SIMD floating-point instructions, 128-bit registers)

S.E.C.C.      = Single Edge Contact Cartridge (SECC1, SECC2)

PPGA          = Plastic PGA (Pin Grid Array)

FCPGA         = Flip-Chip PGA

MPS           = Multi-Processor System

P.I.ROM       = Processor Information ROM

TLB           = Translation Lookaside Buffer

(small cache for page translation)

PnP           = Plug-and-Play

ESCD          = Extended System Configuration Data

DMI           = Desktop Management Interface

ACPI          = Advanced Configuration and Power Interface

APM           = Advanced Power Management

VESA/VBE      = Video Electronics Standards Association /

Video BIOS Extensions

DDC           = Display Data Channel

(DDC1: Monitor->PC, DDC2B: Monitor<->PC)

## 17. Codenames #############################################################

Codenames for Intel Processors:

-------------------------------

P5          - Intel Pentium(R) Processor:

Old 50/50 (Engineering Sample), 60/60 and 66/66 MHz.

P54C        - Intel Pentium(R) Processor:

Later 75/50, 90/60, 100/50, 100/66, 120/60, 133/66, 150/60,

166/66, 180/60, 200/66 and 233/66 MHz without VRT, with

on-chip APIC, Dual Processing Features and FRC support.

P54CM       - Intel Pentium(R) Update Processor:

Old 90MHz Slave processor in Dual Processor system.

P54CQS      - Intel Pentium(R) Processor:

75/50, 100/66, 120/60, 133/66, 166/66 and 200/66 MHz

without VRT, on-chip APIC, Dual Processing Features, Upgrade

and FRC support.

P54LM       - Intel Pentium(R) Mobile Processor:

75/50, 120/60, 133/66 and 150/60 Low Power versions (2.9 V)

supporting VRT. These Processors are shipped without APIC

on-chip, Dual Processing Features, Upgrade and FRC support.

P24T        - Intel Pentium(R) OverDrive(R) Processor:

PODP5V63 63/25 MHz and PODP5V83 83/33 MHz OverDrive

for 486 CPUs.

P24CT       - Intel Pentium(R) OverDrive(R) Processor:

OverDrive for IntelDX4(TM) Processor.

P5T         - Intel Pentium(R) OverDrive(R) Processor:

PODP5V120 120/60 MHz and PODP5V133 133/66 MHz OverDrive for

Intel Pentium(R) P5 60/60 and 66/66 MHz Processors.

P54CT       - Intel Pentium(R) OverDrive(R) Processor:

PODP3V125 125/50 MHz, PODP3V150 150/60 MHz and

PODP3V166 166/66 MHz OverDrive for Intel Pentium(R) P54C

75/50, 90/60 and 100/66 MHz Processors.

P55C        - Intel Pentium(R) Processor with MMX(TM) Technology:

166/66, 200/66, 233/66 and 266/66 MHz

MMX, 32 KBytes First Level Cache and better Branch Prediction.

P55LM

Tillamook   - Intel Mobile Pentium(R) Processor with MMX(TM) Technology:

200/66, 233/66, 266/66, 300/60 and 300/66 MHz,

VccCORE=1.6-2.1 V, VccIO=2.3-2.6 V

P55CTP      - Intel Pentium(R) OverDrive(R) Processor with MMX(TM) Technology:

PODPMT60X150 125/50, 150/60, PODPMT66X166 166/66,

PODPMT60X180 180/60 and PODPMT66X200 200/66 for Intel Pentium(R)

75/50, 90/60 and 100/66 Processors.

P6          - Intel Pentium(R) Pro Processor:

133/66 (Engineering Sample), 150/60, 166/66, 180/60 and 200/66,

256K, 512K and 1024K of L2 cache.

P6T         - Intel Pentium(R) Pro OverDrive(R) Processor.

P6L

Klamath     - Intel Pentium(R) II Processor:

233/66, 266/66 and 300/66,

P6 with MMX(TM) Technology and without Level-2 on-chip cache,

S.E.C. cartridge, Slot 1, 512K half speed ECC/Non-ECC L2

cache on substrate, .35 �m, VccCORE=2.8V or 1.6V,

up to 2 CPUs in MPS

Deschutes   - Intel Pentium(R) II Processor:

333/66, 350/100, 400/100 and 450/100 MHz,

512K ECC half speed L2 cache, MMX, .25 �m, VccCORE=2.0V

Tonga       - Mobile Intel Pentium(R) II Processor:

233/66, 266/66, 300/66, VccCORE=1.7 or 1.6V,

512K ECC half speed L2 cache

P6X         - Intel Pentium(R) II Xeon(TM) Processor:

400/100 and 450/100 MHz, 512K, 1M or 2M o full speed L2 cache,

MMX, Slot 2, up to 8 CPUs in MPS, full ECC, FRC,

SMBus (thermal sensor, P.I.ROM, scratch EEPROM)

P6C

Covington   - Intel Celeron(TM) Processor:

266/66 and 300/66 MHz, no L2 cache, MMX, no MPS, Slot 1

Mendocino   - Intel Celeron(TM) Processor:

300A/66, 333/66, 366/66, 400/66, 433/66, 466/66,

500/66 and 533/66 MHz,

128K on-chip L2 cache, Slot 1 or PPGA (Socket 370)

?           - Intel Mobile Celeron(TM) Processor:

Intel Celeron(TM) Processor Mobile Module:

233/66, 266/66, 300/66, 333/66, 366/66, 400/66, 433/66 and

466/66 MHz, 128K on-chip (full speed) L2 cache, BGA/Micro-PGA

or MMC-1/MMC-2 (Mobile Module) package, Vcc_core=1.5,1.6,1.9 V

?           - Intel Mobile Celeron(TM) Processor:

Intel Celeron(TM) Processor Mobile Module:

400/100, 450/100, 500/100, 550/100, 600/100, 650/100,

700/100 and 750/100 MHz, 128K on-chip (full speed) L2 cache,

BGA2/Micro-PGA2 or MMC-2 (Mobile Module) package,

Vcc_core=1.35-1.6 V

Dixon       - Intel Mobile Pentium(R) II Processor:

Intel Pentium(R) II Processor Mobile Module:

266PE/66, 300PE/66, 333/66, 366/66 and 400/66 MHz,

256K on-chip (full speed) L2 cache, VccCORE=1.5-1.6 V,

Mini-cartridge/BGA or MMC-1/MMC-2 (Mobile Module) package

Katmai      - Intel Pentium(R) III Processor:

450/100, 500/100, 550/100, 600/100, 533B/133 and 600B/133 MHz,

ISSE, 512K half speed L2 cache, Slot 1, Processor Number

Tanner      - Intel Pentium(R) III Xeon(TM) Processor:

500/100 and 550/100 MHz, ISSE, Slot 2, 512K/1M/2M of full

speed L2 cache, thermal sensor, P.I.ROM and scratch EEPROM,

Processor Number

Coppermine  - Intel Pentium(R) III Processor:

500E/100, 533EB/133, 550E/100, 600E/100, 600EB/133, 650/100,

667/133, 700/100, 733/133, 750/100, 800/100, 800EB/133,

850/100, 866/133, 900/100, 933/133, 950/100, 1.0B GHz/133

and 1.13 GHz/133, .18 �m, Slot 1/SECC2 or FC-PGA/Socket370,

256K on-chip full-speed L2 cache, 100/133 MHz FSB,

VccCORE=1.65/1.7 V

Coppermine  - Intel Celeron(TM) Processor:

-128          500A/66, 533A/66, 566/66, 600/66, 633/66, 667/66, 700/66,

733/66, 766/66 and 800/100 MHz, 128K on-chip ATC L2 cache,

PPGA (Socket 370), SSE, Coppermine core

?           - Mobile Intel Pentium(R) III Processor:

400(F)/100, 450(F)/100, 500(F)/100, 600-500(S)/100,

650-500(S)/100, 700-550(S)/100, 750-600(S)/100, 800-650(S)/100,

850-700(S)/100, 900-700(S)/100 and 1000-700(S)/100 MHz,

BGA2, Micro-PGA2 or MMC2, 256K L2 cache,

(S) - SpeedStep Technology

Cascades    - Intel Pentium(R) III Xeon(TM) Processor:

600/133, 667/133, 700/100, 733/133, 800/133, 866/133,

933/133 MHz and 1.0 GHz/133 with 256K full speed ATC L2 cache;

700/100 with 1M and 2M of ATC, Slot 2, .18 �m

Timna       - CPU + Memory controller + Graphics chip - CANCELLED

Tualatin    - Intel Pentium(R) III Processor (?)

.13 �m, 512K L2 ATC, Differntial clocking, AGTL (1.2VTT),

1.26 GHz

Willamette  - Intel Pentium(R) 4 Processor:

.18 �m: 1300/400, 1400/400, 1500/400 and 1700/400 MHz,

Socket 423, Quad pumped FSB (400 MHz),

NetBurst architecture: SSE-2, L1 Instruction Trace Cache,

256K of ATC L2 cache, Rapid Execution Engine, 1.7 V

Features: built-in ACPI, Thermal Monitor

Foster      - Intel Xeon(TM) Processor

1400/400, 1500/400, 1700/400, SMT (Simultaneous Multi-Threading),

256K L2 cache, 0-2MB L3 cache, Socket603

Northwood   - Intel Pentium 4 (?)

.13 �m, copper interconnectors, 1.3 V, mPGA478,

512K of ATC L2 cache

Prestonia   - Intel Pentium 4 Xeon (?)

.13 �m, copper interconnectors

Merced      - Intel Itanium(TM):

IA-64 EPIC architecture, IA-32 and HP PA-RISC compatible,

starting at 733 MHz, "engineering sample"

McKinley    - .18 �m, IA-64, wide-production of Itanium

Madison     - .13 �m

Deerfield

Codenames for AMD Processors:

-----------------------------

Krypton     - AMD-K5

SSA/5 were the first chips with some features disabled

75/50 PR75, 90/60 PR90, 100/66 PR100 - SSA/5 (0.5 �m)

100/66 PR133, 116/66 PR166 - Am5k86 (0.35 �m)

VccCORE=3.3 V, Socket 7

K6          - AMD-K6

166/66, 200/66, 233/66 (0.35 �m), 266/66, 300/66 (0.25 �m)

MMX, Socket 7, VccCORE=2.9 or 3.2 or 2.2 V

K6-2        - AMD-K6-2

266/66 AFR, 300/100 AFR, 333/95 AFR, 350/100 AFR, 366/66 AFR,

380/95 AFR, 400/100 AFQ, 400/100 AFR, 450/100 AHX, 450/100 AFX,

475/95 AHX, 475/95 AFX, 500/100 AFX, 533/97 AFX, 550/100 AGR

MMX, 3DNow!, Super7 Socket, VccCORE=2.2V (AFR, AFQ, AFX) or

2.3V (AGR) or 2.4V (AHX), 0.25 �m

Sharptooth  - AMD-K6-III

400/100 AHX, 400/100 AFR, 450/100 AFX, 450/100 AHX,

256K of On-chip integrated cache, 0.25 �m,

VccCORE=2.2V (AFR, AFX) or 2.4V (AHX)

K7          - AMD Athlon

500/200 A/C, 550/200 A/C, 600/200 A/C, 650/200 A/C, 700/200 A/C,

750/200 A, 800/200 A, 850/200 A, 900/200 A, 950/200 A and

1000/200 A MHz, 0.25 (C) and 0.18 (A) �m, 1.6 V,

512K of L2 cache running at 1/2 (500-700), 2/5 (750-850) or

1/3 (900-1000) of core speed, MMX, 3DNow!, Slot A

Thunderbird - AMD Athlon

integrated 256K of full-speed L2 cache, .18 �m, Copper, 1.75 V,

Lightning Data Transport, 200 MHz DDR FSB, Slot A/Socket A,

650/200, 700/200, 750/200, 800/200, 850/200, 900/200, 950/200,

1000/200, 1100/200, 1133/133, 1200/200, 1300/200 and 1333/133 MHz

Corvette    - Mobile Athlon

Spitfire    - AMD Duron

.18 �m, 64KB integrated L2 cache, 200 MHz FSB, Socket A, 1.6 V,

550/200, 600/200, 650/200, 700/200, 750/200, 800/200, 850/200

and 900/200 MHz

Palomino    - AMD Athlon 4

.18 �m, 256KB integrated L2 cache, MPS support, SSE, enhanced TLB, thermal diode,

850/200, 900/200, 950/200 and 1000/200 MHz

Morgan      - AMD Duron

Appaloosa   - .13 �m

Thoroughbred - .13 �m

Barton      - .13 �m, SOI

ClawHammer   - x86-64, 1-2 way MP, .13 �m

SledgeHammer - x86-64, 4-8 way MP, .13 �m

## 18. Thanks ################################################################

I want especially to thank:

� Vladimir Vylicil <lada.vyl@atlas.cz> - great testing and reporting !!!

� Alex V. Potemkin and Potemkin's Hackers Group <avp@stelth.dvm.msk.ru>

� Aad Offerman <offerman@einstein.et.tudelft.nl> - CHIPLIST

� J�n G�cik <jan.gucik@vslib.cz> - IntroScope

� Tom�s Lehuta <lehuta@elf.stuba.sk> - GUS Memory routs, hints

� Peter Hubinsky <hubak@elf.stuba.sk; SAC fileadmin> - distribution

� Emil Laurentiu <lcarmen@u1.ici.ro> - hints

� Andrew Nelson <anelson@atlas.co.uk> - English language suggestions :-)

� Marek Smihla <ceit@internet.sk> - reports & hints

� Jiang Hong <zlzhang@pku.edu.cn> - BL3 & hints

� Intel's support <support@intel.com> - excellent dox, insight CD, support

� Tamas Miklos <fieryofup@geocities.com> - WSS&Modem rout, hints, ASMDEMO

� ESCOM CZ <escom_support@brn.pvtnet.cz> - info, reports

� Olivier Cahagne <cahagn_o@epita.fr> - SysInfo observatory, reports

� Christian Klak <drk-soft@gmx.net> - Acer info

� Gershon Mikulsky <mikuls@shani.net> - ATI info

� EliCZ <elicz@post.cz> - AMD-K6 DRx bug

� ATI Developer Relations <devrel@atitech.com> - id of ATI chips

� Vasil Tonev <mi@iterra.net> - Web Site

� Brian Richardson <brianr@ami.com> - info

� Jonathan Teh Soon Yew <j.teh@iname.com> - MBPROBE, info

� Vincent Chan <vchan@ati.com> - ArtX info

� Reports:

- Janusz Bubik <bubik@levi.cz>

- Thomas Mainka <100112.2132@compuserve.com>

- George Solonaru <georges@mail.dntis.ro>

- The-wanderer <the-wanderer@home.com>

- Silviu Guseila <silviug@pcnet.ro>

- Vladimir Afanasiev <dxover@tcms12.rt.ru>

- Tomek Borukalo <Borukalo@CHIP.VOGEL.PL>

- Frank Stephan <internettreff@gmx.de>

- Andrzej Marcinczyk <amarc@pg.gda.pl>

- Walter Co <walter@asiantech.com>

- Tomy Chen <tomychen@tomail.com.tw>

� Beta testers: Boris Hur�nek [BHsoft] <hurinek@axa.sk>

Erik Francisci <jumper!jumper@jumper.sk>

Igor Junas <junas@axis.sk>

Mat�s Rosa <rosa1@post.sk>

Milan Marusinec <marusin@lithec.sk>

� All other people, who have sent bug reports and suggestions (*!*@*.*)

Greetings to:

� Lubos Vrtik <vrtik@vuje.sk> - cracking <-> protecting impressions, HMVS

� C00LMAN [Siam Hacker's Gang] - KeY GeNeRaToR 4 HWiNFO v<4.0.0

I'm waiting 4 keYGen 4 HWiNFO v4.0.0+

� eGIS [CORE] - *v3Ry nICE* K3YG3N 4 HWiNFO �4.2.4, 4.5.4, ...

Let me know when you forget HWiNFO :-)

No freeware in the near future.

� EliCZ [pCE] - {Ni<E W<>rK} - keygen 4 HWiNFO <4.4.4

A little bit complicated, but at least something new.

� dEVIL [uCF] - *nICE* kEYZ 4 HWiNFO v4.2.2, ReGGeD v4.3.0

Brute force rulez...

� Dark Paw - Nice Try (also w/ comments)

Still waiting for registered users ?

� lorian [uCF] - Superior work :-)

No comments... You'll see...

� All unknown members of HWiNFO <Ra<KiNG challange :-)

## 19. Disclaimer ############################################################

����������������������������������������������������������������������������ͻ

�  All product names are copyright and registered trademarks/tradenames of   �

�                        their respective owners.                            �

����������������������������������������������������������������������������ͼ

HWiNFO IS COPYRIGHTED MATERIAL AND PROTECTED BY APPLICABLE

COPYRIGHT LAWS AND INTERNATIONAL TREATY PROVISIONS.

THE PROGRAM AND ITS DOCUMENTATION ARE COPYRIGHTED BY THE ORIGINAL

PROGRAMMER/AUTHOR (COPYRIGHT HOLDER). ALL RIGHTS ARE RESERVED CONCERNING

FUTURE CHANGES OF THE NAME AND CONTENTS OF THIS DISTRIBUTION PACKAGE. NO

CHANGES MUST BE MADE FOR ANY REASONS AT ANY TIME BY ANYBODY IN ANY ASPECT,

INCLUDING, BUT NOT LIMITED TO, MODIFICATION OF THE COPYRIGHT NOTICE OR

CHANGES OF THE COMPILED BINARIES HEREBY ALTERING THE ORIGINALLY INTENDED

PROGRAM EXECUTION FLOW.

REALiX SPECIFICALLY DISCLAIMS ALL WARRANTIES, EXPRESS OR IMPLIED, AND

ALL LIABILITY, INCLUDING CONSEQUENTIAL AND OTHER INDIRECT DAMAGES, FOR

THE USE OF THIS SOFTWARE, INCLUDING LIABILITY FOR INFRINGEMENT OF ANY

PROPRIETARY RIGHTS. REALiX DOES NOT ASSUME ANY RESPONSIBILITY FOR ANY

ERRORS WHICH MAY APPEAR IN THIS CODE NOR ANY RESPONSIBILITY TO UPDATE IT.

THE PERSON USING THE SOFTWARE BEARS ALL RISKS AS TO THE QUALITY AND

PERFORMANCE OF THIS SOFTWARE.

LICENSE

=======

SHAREWARE COPY

--------------

A shareware (unregistered) copy of HWiNFO is the distributed package

of HWiNFO without any key file. This copy may be distributed freely,

but must not  be modified in any way.  It is not allowed to sell the

shareware copy of HWiNFO like a standalone program; a shareware copy

is free.

REGISTERED COPY

---------------

A registered copy includes the shareware copy plus a valid key file.

A user  may use  the registered copy of HWiNFO  only when  accepting

the terms of license. It is not allowed to distribute this copy.

LICENSE TYPES

-------------

A non-commercial license means, that the user may use the registered

copy  for  non-commercial  purposes  only  (e.g.  for own using, for

education, etc.). A commercial license enables the license holder to

use the registered copy of HWiNFO for commercial purposes also.

LICENSE AMOUNT

--------------

The license amount  selects the  amount of licensed copies, that are

allowed to run within  the registered subject. A single-user license

allows the subject  to use  only  one registered copy at a time.  An

unlimited license enables the license holder  to register any number

of copies of HWiNFO using the given key file.

TERMS OF LICENSE

----------------

A key file may be used  ONLY  by the  appropriate registered subject

and  only in such  a way, that doesn't  violate the license type and

license amount limitations.  The name of license holder is displayed

in the first dialog after running HWiNFO (the "Registered to" line).

The license type and amount is displayed too.

A subject is using the program legally, only when accepting and not

violating the terms of license.

It is not allowed to:

- sell the shareware copy of HWiNFO like a standalone program

- modify a copy of HWiNFO or the key file

- give the key file to person, which isn't granted to use it

- distribute or sell the key file

- violate terms of license

## END #######################################################################